Technical Report

Automation of NDOT Concrete Mix Design Model

Client: Nebraska Department of Transportation (NDOT)
Course: CIVE 202 – Spring 2026

1. Introduction

Currently the Nebraska Department of Transportation uses an Excel summary worksheet with functions and a set of rules to calculate the concrete mix for 1 cubic yard of concrete. Although they have met their requirements, the Excel summary worksheet is subject to accidental modifications to the formulas; there are many hidden cell dependencies; the scalability of these worksheets is limited. To improve reliability, repeatability, and the potential of future integration, NDOT engaged in the development of an automated Python-based mix design calculator using the engineering logic inherent in the NDOT 'Mix Design' Excel worksheet.

The result will be a modular system that will translate the logic of the Excel worksheet into a Python system which allows the user to provide sequential inputs, and the system produces an automated output. The output will consist of a summary of the total weight of concrete for 1 cubic yard of concrete, as well as four valid realistic concrete mix designs for validation purposes.

2. Methods

The Excel workbook contained embedded formulas calculating:

- Water weight from total cementitious materials and water–cement ratio

- Material volumes using specific gravity relationships

- Air volume based on target percentage

- Remaining aggregate volume using total cubic yard volume balance

- Aggregate weights using specific gravity and proportional allocation

These formulas were translated into clearly defined Python functions in the Jupyter Notebook.

Excel Logic (Conceptual Form):

- Water Weight = (Cement + Fly Ash + Silica + Other SCM) × W/C Ratio

- Volume = Weight ÷ (Specific Gravity × 62.4)

- Aggregate Volume = 27 ft³ – (Sum of all material volumes)

Python Implementation:

- water_weight() function calculates total water demand

- cement_volume(), flyash_volume(), etc., convert weights to volumes

- AggregateVolume() enforces total cubic yard balance

- Aggregate weight functions compute fine, coarse, and other aggregate weights

By isolating each calculation into modular functions, the Python system improves transparency and reduces the risk of accidental formula corruption.

Sequential User Input Structure

- Cementitious material weights

- Water–cement ratio

- Target air percentage

- Aggregate proportions

- Specific gravities

This mirrors the Excel worksheet structure while ensuring a structured and repeatable workflow.

Engineering Assumptions

- Total concrete volume fixed at 1 cubic yard (27 ft³)

- Water density assumed constant at 62.4 lb/ft³

- Aggregate percentages distribute remaining volume

- No moisture corrections applied

- No scaling beyond 1 cubic yard

These assumptions match the structure of the original Excel-based design framework.

3. Results and Discussion

We used the automated python model to examine a total of four unique realistic concrete mix scenarios. The only difference in the four mixes was the amount of cement, water and cement ratio, amount of air, and the proportions of aggregate.

Trends

1. Water/Cement Ratio

Higher water/cement ratio mixes produced heavier aggregate weights due to the proportional increase in the amount of water in the mix. Therefore, the total amount of water in the mix increases, but there is less available aggregate volume because the mix is still limited to the 27 cubic foot (ft³) total volume constraint. As the water/cement ratio is decreased, the aggregate will be packed more tightly together and there will be less amount of water.

2. Influence of Air Content

Higher target percentages of air reduce the amount of aggregate since the air volume is taken as a percentage of the total cubic yard volume. As a result, increasing the amount of air in the mix will decrease the amount of volume available to aggregates. This was particularly evident in mixes designed for freeze-thaw durability.

3. Variations in Cementitious Material

The presence of supplementary cementitious materials such as fly ash and silica fume will change the total weight of the cementitious material in the mix and, therefore, the total amount of water required for the mix. Mixes containing high levels of silica had higher paste volumes.

Functional Validation

Although no formal numerical comparisons were made in relation to excel outputs, validation occurred by:

- Replication of the formula structure in excel

- Unit testing of individual functional components

- Conducting four independent runs through the same scenario

- Validation of the volume balance

All scenarios ran to completion with no errors in calculations and produced results that met expectations with respect to engineering standards indicating that the logic was transferred correctly.

4. Benefits of Python Automation

Advancing from Excel to Python offers several advantages:

Improved reliability.

Excel formulas can be edited or incorrectly referenced; Python functions have the calculation logic bundled into predefined modules.

Transparency.

Each function contains a clear definition of engineering relationships, which enhances traceability.

Repeatability.

Sequential user input provides standardized data entry, which results in lower levels of human error.

Scalability.

Python implementation can be integrated into: Web-based design systems Larger-scale NDOT automation systems Batch processing.

5. Limitations

- No moisture adjustments

- No automated durability exposure classifications

- Limited to 1 cu yd

- No graphics interface

Potential future work may incorporate automated NDOT Class settings, moisture corrections, and web-based integration.

6. Conclusion

The NDOT automated the concrete mix design system by creating a modular Python-based system that replaced the previously used Excel spreadsheet-based mixture design model. This system replicates all basic engineering computations as well as providing a professionally produced weight summary for 1 cubic yard of concrete.

Four mixes were tested for functionality and engineering behavior, with consistent performance in all cases. By moving to Python, NDOT will have greater reliability, repeatability, and a greater ability to integrate in the future.

Automating this process will provide more transparency of engineering practices and move the engineering mix design workflow closer to meeting the Digital Infrastructure objectives of NDOT.

7. References

Nebraska Department of Transportation (NDOT). Concrete Mix Design Submittal Workbook. Retrieved from: https://dot.nebraska.gov/media/jp3paote/mix-design-submittal.xlsx

Project 2 - Client Prompt

Nebraska Department of Transportation (NDOT). Project #2: Automation of Concrete Mix Design – Client Prompt.