4 Mix Scenarios

Scenario 1: Normal-Strength Structural Concrete (≈ 4000 psi)

Typical slabs, footings, beams

Target Amounts (lb/yd³)

- A – Cement: 650

- B – Fly Ash: 100

- C – Silica Fume: 0

- D – Other SCM: 0

- E – Target w/c: 0.45

- F – Air Content (%): 4.5

- G – % Fine Aggregate: 40

- H – % Coarse Aggregate: 60

- I – % Other Aggregate: 0

Specific Gravity

- J – Cement: 3.15

- K – Fly Ash: 2.40

- L – Silica Fume: —

- M – Other SCM: —

- N – Fine Agg: 2.65

- O – Coarse Agg: 2.70

- P – Other Agg: —

Scenario 2: High-Strength Concrete (≈ 6000 psi)

Columns, bridge decks

Target Amounts

- A – Cement: 800

- B – Fly Ash: 120

- C – Silica Fume: 40

- D – Other SCM: 0

- E – Target w/c: 0.32

- F – Air Content: 3.0

- G – % Fine Agg: 38

- H – % Coarse Agg: 62

- I – % Other Agg: 0

Specific Gravity

- J – Cement: 3.15

- K – Fly Ash: 2.40

- L – Silica Fume: 2.20

- M – Other SCM: —

- N – Fine Agg: 2.65

- O – Coarse Agg: 2.70

- P – Other Agg: —

Scenario 3: Air-Entrained Pavement Concrete (≈ 4500 psi)

Roadways, freeze-thaw exposure

Target Amounts

- A – Cement: 600

- B – Fly Ash: 150

- C – Silica Fume: 0

- D – Other SCM: 0

- E – Target w/c: 0.42

- F – Air Content: 6.5

- G – % Fine Agg: 42

- H – % Coarse Agg: 58

- I – % Other Agg: 0

Specific Gravity

- J – Cement: 3.15

- K – Fly Ash: 2.40

- L – Silica Fume: —

- M – Other SCM: —

- N – Fine Agg: 2.64

- O – Coarse Agg: 2.70

- P – Other Agg: —

Scenario 4: Sustainable / High Fly Ash Concrete (≈ 3500 psi)

Mass concrete, foundations, low heat

Target Amounts

- A – Cement: 500

- B – Fly Ash: 250

- C – Silica Fume: 0

- D – Other SCM: 0

- E – Target w/c: 0.48

- F – Air Content: 4.0

- G – % Fine Agg: 45

- H – % Coarse Agg: 55

- I – % Other Agg: 0

Specific Gravity

- J – Cement: 3.15

- K – Fly Ash: 2.35

- L – Silica Fume: —

- M – Other SCM: —

- N – Fine Agg: 2.63

- O – Coarse Agg: 2.68

- P – Other Agg: —